Table of Contents

# Non-rectangular grids for OSMOSE-Python — feasibility study

Status: exploration (2026-04-24). No implementation committed. Purpose: map what it would take to support hexagonal or unstructured (FEM-style) grids, identify engine changes, estimate cost per subsystem, and recommend a migration path. Scope: the Python engine, config schema, outputs, and UI. The Java engine is not in scope.

## TL;DR

OSMOSE-Python today is hard-wired to a rectangular (nlat × nlon) grid with row-major indexing and 8-connectivity via integer (dr, dc) arithmetic. The engine already separates cell-local logic (predation, forcing, fishing) from cell-topology logic (movement, neighborhood, output writing). That separation makes three generalization paths technically possible, in increasing order of engineering cost:

- Curvilinear rectangular grid (still (ny, nx), but distorted cells — e.g., a polar or tripolar ocean grid). Cost: LOW–MEDIUM. Already partially supported via NcGrid which reads 2D latitude/longitude arrays. Only output-writing and UI polygon construction need rework.

- Regular hexagonal grid. Cost: MEDIUM. Re-indexing (axial or offset coordinates), 6-connectivity instead of 8-, new random-walk kernel, new UI polygons. Predation/forcing/fishing unaffected once cell IDs are abstracted.

- Unstructured mesh (triangles or arbitrary polygons, FEM-style). Cost: HIGH. Grid is a graph; every (dr, dc) arithmetic disappears; movement maps become per-cell arrays indexed by cell ID; a substantial config+I/O+UI reworking is required.

The highest barriers across all three options are (a) random-walk movement, (b) map-based movement CSV format, and (c) UI rendering. The easiest are predation, forcing, and per-cell fishing — they are already cell-local.

Recommendation: introduce a GridTopology abstraction now (even if still rectangular under the hood) to isolate the cells-with-arithmetic assumption. Then iterate: curvilinear first (nearly free), hex second, FEM as a longer-term research track.

## 1 · Why consider non-rectangular cells?

Four ecological / numerical motivations that come up periodically for OSMOSE-type models:

- Coastline fidelity. A 0.4° × 0.3° rectangular grid on the Baltic produces 1,384 land cells and 616 ocean cells; the coastline is blocky and a ~25 km coastal strip often straddles a single cell. Finer rectangular grids would quadruple the cell count for a linear improvement. Hex or adaptive meshes can resolve coastline at higher effective resolution for a given cell budget.

- Uniform neighbor distance. Rectangular 8-connectivity has √2 distance asymmetry between axial and diagonal neighbors. Hexagonal 6-connectivity has uniform nearest-neighbor distance, which makes dispersal kernels (random walk) interpretable without correction.

- Curvilinear coordinates. High-latitude ocean models (ROMS, NEMO) use curvilinear grids to avoid the pole singularity. Forcing from such models currently has to be regridded onto OSMOSE’s rectangular grid, losing mass-conservation properties.

- Regional refinement. FEM/unstructured meshes let you put high resolution where it matters (coast, river mouths, spawning grounds) and coarse resolution offshore. For Baltic-OSMOSE, this would let us resolve Vistula Lagoon, Curonian Lagoon, Pärnu Bay coastal nurseries at fine scale without blowing up the ocean-basin cell count.

These are not enough to justify the change on their own. But they matter for Baltic-like setups where coastal resolution is the limiting factor.

## 2 · What the code assumes today (survey results)

Based on a codebase survey conducted 2026-04-24:

| Subsystem | File | Grid assumption | Generalization cost |
| --- | --- | --- | --- |
| Grid representation | osmose/engine/grid.py:16-122 | 2D (ny, nx) arrays; row-major 1D via cell_id = y*nx + x; 8-connectivity by for dy, dx in {-1,0,1}² | MEDIUM |
| Random walk movement | osmose/engine/processes/movement.py:163-193 | Cartesian L∞ displacement; walk_range is an integer bounding box | HIGH |
| Map-based movement | osmose/engine/processes/movement.py:33-103 | CSV is (ny, nx) matrix; rejection-samples from 2D probability grid; BFS patch uses grid.neighbors() | HIGH |
| Predation | osmose/engine/processes/predation.py | Per-cell matching; no neighbor interaction | LOW |
| LTL/background forcing | osmose/engine/resources.py, osmose/engine/background.py | Flat per-cell biomass; time interpolation only; NetCDF loaded by (time, lat, lon) | LOW |
| Fishing spatial maps | osmose/engine/config.py:1417-1436 | CSV as (ny, nx); per-cell rate lookup | LOW |
| Non-spatial output | osmose/engine/output.py:23-91 | None | — |
| Spatial output | osmose/engine/output.py:492-640 | NetCDF dims (time, species, lat, lon); rectangular coordinates | MEDIUM |
| UI overlay rendering | ui/pages/grid_helpers.py:289-518, 627-707 | Rectangular polygons from meshgrid; 4-corner construction | HIGH |
| Mask | ui/pages/grid_helpers.py:117-138, osmose/engine/grid.py:74-115 | Per-cell boolean, CSV (ny, nx) or NetCDF 2D | MEDIUM |
| Config schema | osmose/schema/grid.py:5-98 | grid.nlon, grid.nlat, bounding-box corners | MEDIUM |

The full-detail survey with file:line references lives in the notes that generated this document.

Key structural observation: the engine already separates cell-local logic from cell-topology logic. Predation, fishing, and forcing never ask “what are my neighbors?” — they read and write per-cell values. Movement is the one place that cares about topology (random walk) and 2D layout (map CSVs). The UI also cares, because it draws polygons.

## 3 · Option A — Curvilinear rectangular grid

What this is: Keep the (ny, nx) 2D indexing. Drop the requirement that cell-center longitudes are equal-spaced; drop the requirement that cells are axis-aligned rectangles. Cell (i, j) has arbitrary lat[i, j], lon[i, j] — the standard curvilinear convention used by ROMS, NEMO, POM, and Copernicus Marine products.

Why it’s cheap. Grid.from_netcdf() at osmose/engine/grid.py:74-115 already reads 2D latitude / longitude arrays. The engine indexes cells by (i, j) pair, not by coordinate. Internal semantics don’t change.

What changes:

- Mask loader (grid.py): already handles 2D lat/lon — no change.

- Movement random walk: still valid if interpreted in (i, j) index space (not physical km). Current code does this. But displacement is now anisotropic in km — cell (i, j) to (i+1, j) may be 5 km in one place and 50 km elsewhere. Users need to know this. Change: add a docstring + optional per-cell km-weighted walk mode.

- Movement map CSV: still valid — each row/column stays an (i, j) index.

- Forcing / fishing: unchanged.

- Spatial output: today’s NetCDF writer assumes 1D lat(ny,) and 1D lon(nx,). For curvilinear grids, needs to write latitude(ny, nx) and longitude(ny, nx) per CF-conventions. Change: ~30 lines in osmose/engine/output.py:492-640.

- UI rendering: build_netcdf_grid_layers in ui/pages/grid_helpers.py:469-518 already handles 2D lat/lon via finite-difference cell edges. Will work for smooth curvilinear grids, may look jagged for highly distorted ones. Change: none needed; later, could add a proper Voronoi-of-centers fallback.

- Config: add a grid.curvilinear = true flag; remove the grid.upleft.* requirement when set.

Rough cost: 1–2 engineer-weeks. Most effort is in the output writer and a test suite verifying a ROMS-style input runs through unchanged. This is the right first step regardless of where we go next — it forces us to stop assuming dx = constant.

## 4 · Option B — Regular hexagonal grid

What this is: All cells are regular hexagons of a fixed size, tiled across the domain. Each cell has exactly 6 neighbors. Indexing uses axial coordinates (q, r) (efficient for storage) or offset coordinates (easier visual mapping). Well-studied libraries: h3 (Uber, millions of users), astar for pathing, pyproj for cell-to-lat/lon projection.

Why hex over rectangular: - Uniform nearest-neighbor distance → dispersal kernels are cleaner. - Natural 6-connectivity matches biological diffusion models better than rectangular 8-connectivity. - H3 has hierarchical nesting (resolution levels 0–15), making mesh refinement straightforward.

What changes:

- Grid class (osmose/engine/grid.py): replace 2D array storage with an explicit cell table:

- class HexGrid:
    cells: NDArray[np.int64]          # shape (n_cells,), the cell IDs
    lat: NDArray[np.float64]          # shape (n_cells,)
    lon: NDArray[np.float64]          # shape (n_cells,)
    ocean_mask: NDArray[np.bool_]     # shape (n_cells,)
    neighbors: NDArray[np.int64]      # shape (n_cells, 6), -1 for missing

- Neighbors are computed once at load time (via h3.k_ring(cell, 1) or direct hex arithmetic). Effort: ~200 LOC new file, or new branch in grid.py.

- Random walk: replace (dr, dc) ∈ {-1,0,1}² with random neighbor selection: next_cell = neighbors[current_cell, rng.integers(0, 6)]. The walk_range parameter becomes a k-ring radius (cells within k steps). Change: ~40 LOC in movement.py:163-193.

- Map-based movement: CSV format changes. Today each row/column is an (i, j) grid cell; for hex, a single column with n_cells rows indexed by cell ID is the minimum viable format. Change: ~80 LOC to support a new CSV schema + back-compat detection.

- Predation / forcing / fishing: no change to logic. They operate on flat cell IDs; today those happen to be y*nx+x, tomorrow they’re H3 indices. Everything that ingests a NetCDF with (time, n_cells) or (time, lat, lon) needs to be told which representation the grid uses. Change: minor — thread the grid representation through a few ingestion functions.

- Spatial output: write UGRID-convention NetCDF with a mesh variable + face_node_connectivity + per-face data vars. xarray + uxarray supports this natively. Change: ~60 LOC in output.py, pick up a dependency (uxarray or write UGRID manually with xarray.Dataset).

- UI rendering: per-cell polygon data precomputed at grid load — 6 vertices per hex from hex-to-geographic projection. polygon_layer in shiny-deckgl accepts arbitrary polygons, so the primitive is already there. Change: ~80 LOC in ui/pages/grid_helpers.py, replacing build_grid_layers with a build_hex_grid_layers branch.

- Config: new key grid.type ∈ {rectangular, hex} plus grid.hex.resolution (0–15 for H3) or grid.hex.size_km. Mask + bathymetry load from per-cell CSV.

Rough cost: 4–6 engineer-weeks. The biggest risk is validating that movement statistics (dispersion rate, home-range size) match the rectangular reference within calibration tolerance. This requires a parity test against a known rectangular case using the same underlying dispersal process.

External dependency: h3 (Apache 2.0) is a one-line add to pyproject.toml. Alternative: hand-rolled axial coordinates (~100 LOC, no dependency).

## 5 · Option C — Unstructured mesh (FEM-style)

What this is: Cells are arbitrary polygons (triangles, quads, Voronoi cells). Topology is a graph — each cell knows its neighbors by ID, there is no coordinate arithmetic. Widely used in coastal ocean models (ADCIRC, FVCOM, SCHISM) and in finite-element fluid dynamics. The standard file format is UGRID CF convention on top of NetCDF.

What changes (on top of Option B):

- Grid class: same general structure as hex (cells, lat, lon, ocean_mask, neighbors), but neighbors is a ragged array (cells can have 3, 4, 5, 6, … neighbors). Store as (cell_ids, neighbor_offsets) CSR-style. Effort: ~300 LOC including a UGRID reader.

- Random walk: same as hex — pick random neighbor. But now walk distance in km depends on cell, so “walk_range=2” is less interpretable. Need to decide: is walk_range in graph steps or km? If km, need a precomputed neighbor-distance matrix (sparse). Change: ~100 LOC.

- Map-based movement: CSV must become per-cell (n_cells-row column). Same as hex.

- Predation / forcing / fishing: same as hex — unchanged logic, flat cell IDs.

- Spatial output: UGRID NetCDF with face_node_connectivity, node_coordinates, etc. xarray does not natively write UGRID; uxarray does. Change: add uxarray dependency or roll our own.

- Mask generation: offline mesh-generation pipeline (gmsh, jigsaw, OceanMesh2D). OSMOSE doesn’t generate the mesh — it consumes one.

- UI rendering: render polygons from UGRID face_node_connectivity. Per-cell polygon count varies. polygon_layer handles this. Change: ~150 LOC.

- Config: new key grid.type = unstructured + grid.mesh.file pointing to a UGRID NetCDF. Remove all nlon/nlat expectations.

- Validation: mesh sanity (closed, no degenerate cells, matches ocean extent) — probably integrate meshio or pyvista for validation.

Rough cost: 3–4 engineer-months. Not because any single piece is hard, but because the surface area is large: mesh reader, mesh QA, UGRID output, non-uniform dispersal kernels, UI polygon rendering, documentation, examples, and retrofitting every existing test fixture. Plus: mesh generation is an out-of-tree problem we don’t want to own.

External dependencies: uxarray (PyPI, LGPL-3.0) for UGRID I/O; optionally meshio for format conversions. Neither is heavy.

## 6 · Cross-cutting engineering concerns

### 6.1 Abstraction layer first

Whatever we do, the cleanest first step is to introduce a GridTopology interface that the engine talks to instead of the current Grid with its (ny, nx) assumptions. Something like:

class GridTopology(Protocol):
    n_cells: int
    ocean_mask: NDArray[np.bool_]        # shape (n_cells,)
    lat: NDArray[np.float64]             # shape (n_cells,)
    lon: NDArray[np.float64]             # shape (n_cells,)
    def neighbors(self, cell_id: int) -> NDArray[np.int64]: ...
    def random_neighbor(self, cell_id: int, rng) -> int: ...
    def walk_k_ring(self, cell_id: int, k: int) -> NDArray[np.int64]: ...

RectangularGrid becomes the first implementation. Engine callers stop using (y, x) tuples and use flat cell IDs. This refactor is purely mechanical — no test should change in outcome — and it exposes every place that currently assumes grid shape. Estimated 2 engineer-weeks for the refactor alone.

After this, new topologies (hex, unstructured) slot in as additional implementations of the GridTopology protocol.

### 6.2 Movement subsystem is the main surgery

The survey pegged movement as HIGH cost on every non-rectangular option. Two specific sub-problems:

- Random walk semantics. “walk_range = 2” today means a 5×5 integer box. On a hex grid it could mean “2-ring” (~19 cells). On an unstructured mesh it could mean “graph distance ≤ 2” or “≤ 2 × cell_size km”. These are biologically different. Each species’ calibrated walk_range value may need rescaling — probably a per-species calibration pass.

- Map-based movement CSV format. Today maps are 2D matrices that align with the rectangular grid. For hex / unstructured, maps become per-cell probability vectors. The engine should keep accepting the 2D format for rectangular grids (back-compat) and add a new “per-cell” format keyed by cell ID. The existing smelt_spawning.csv and friends would then need regeneration for the new topology — reusing the HELCOM-literature-derived cell lists from the 2026-04-21 map-validation session.

### 6.3 Mass conservation and bathymetry

OSMOSE treats cells as abstract “schools live here” containers, not physical volumes. So strictly speaking, cell area doesn’t enter the biomass arithmetic — biomass per cell is an abundance not a density. Two places where area would matter:

- Forcing with density units. If LTL biomass were in mg/m³ instead of tonnes/cell, cell area matters. Today it’s the latter, so this isn’t an issue.

- Fishing rate spatial aggregation. Fishing is per-cell with a mortality rate; if effort is expressed as boats/km², you need cell area.

Neither is currently a problem but will surface if OSMOSE tries to couple with physical/biogeochemical models more tightly.

### 6.4 Bathymetry / depth

OSMOSE-Python doesn’t currently use depth at all. That’s orthogonal to the grid-topology question but would become relevant if we add depth-stratified predation (cod in deep basins, flounder on the shelf). For unstructured meshes with depth, you might want voxel-level meshes (rare, but 3D versions of SCHISM exist). Deferred.

### 6.5 Calibration compatibility

All existing Baltic calibration parameters (mortality rates, fishing rates, etc.) are per-species scalars — they don’t depend on grid topology. So a topology switch doesn’t invalidate the 2026-04-22 phase 1 or 2026-04-24 phase 2 calibration as a starting point. Species-distribution maps (smelt_spawning.csv etc.) would need regeneration, not recalibration.

### 6.6 Parity with the Java reference

The Java OSMOSE engine is the reference implementation. It’s also rectangular-only (as of the versions we track). Switching Python to a non-rectangular topology breaks numerical parity with Java. This is an explicit non-goal for this feature — it would fork the Python engine from being a parity port to being a divergent evolution. That’s a strategic decision, not a technical one.

### 6.7 UI performance

shiny-deckgl renders polygons with GPU acceleration. 10,000 hex cells renders fine in our testing. 100,000+ unstructured cells may stutter; mitigate by viewport culling or switching to bitmap_layer for very fine meshes. Not a blocker at OSMOSE-scale domains.

## 7 · Migration path — recommendation

If this is worth pursuing, the low-regret sequence is:

- Phase 0 — Abstraction (2 weeks). Introduce GridTopology protocol; port RectangularGrid to implement it; refactor every caller to use flat cell IDs; existing test suite still passes byte-for-byte. No functional change, but the change-point becomes visible.

- Phase 1 — Curvilinear NcGrid validation (1 week). End-to-end test of running OSMOSE-Python against a ROMS-style 2D (ny, nx) grid with distorted cells. Validate spatial output writes CF-compliant curvilinear NetCDF. Deliver: first non-trivial non-uniform-dx grid running through the engine.

- Phase 2 — Hex grid (4–6 weeks). Implement HexGrid on top of h3. Validate: run a Baltic-shaped hex grid through the existing calibration pipeline. Compare biomass trajectories against the rectangular-grid equivalent. If within calibration tolerance, hex is viable. Re-derive Baltic distribution maps on hex and re-run calibration.

- Phase 3 — Unstructured mesh (3–4 months). Only if Phase 2 proves hex is worth the modeling effort. Implement UnstructuredGrid reading UGRID NetCDF. Use Baltic as a test bed with a mesh refined in coastal lagoons. Major downstream consequence: every distribution map needs a mesh-aware regeneration. Publish as a research track, not a stable feature.

Total horizon: ~6 months if we commit fully; ~3 weeks for Phase 0+1 alone, which already delivers real value (proper curvilinear grid support) and opens the door to later phases without prejudging them.

## 8 · What we would not get from this

Worth stating explicitly so the scope stays honest:

- Not more accurate ecology without recalibration. All species distribution maps + possibly walk_range would need regeneration for the new topology. The gain is spatial resolution where it matters, not automatic model improvement.

- Not Java parity. Java OSMOSE is rectangular. Any non-rectangular Python grid forks from Java.

- Not mesh generation. OSMOSE consumes meshes; we rely on gmsh / jigsaw / OceanMesh2D upstream. Our grid files are inputs.

- Not solving the Baltic biomass-calibration gap. That’s a parameter-fitting problem (see 2026-04-24 Tier 1 plan), not a grid-resolution problem. A finer grid would expose coastal heterogeneity that the model doesn’t currently represent, but the out-of-range biomasses at ×100+ target aren’t caused by grid coarseness.

## 9 · Open questions

- Is Java-parity breakage acceptable? (Strategic, needs project-level decision.)

- Do we need simultaneous support for rectangular + non-rectangular, or a clean migration where only one topology exists at a time? (Config schema implication.)

- Are there existing UGRID-format Baltic meshes we can reuse (e.g., from HELCOM bathymetry work)?

- Is hex H3 at resolution 5 (~8.5 km²) or 6 (~1.2 km²) the right scale for Baltic? (h3 resolution choice).

- Would SIMD-friendly layouts (StructureOfArrays of cell attributes) give enough speedup to offset the indirection cost of a neighbor graph? (Performance spike needed.)

## References

- Survey notes behind this document: generated by an Explore subagent on 2026-04-24, documenting file:line references for every grid-assumption in the codebase.

- UGRID CF convention: https://ugrid-conventions.github.io/ugrid-conventions/

- H3 geospatial indexing: https://h3geo.org/

- uxarray: UGRID-aware xarray extension, NSF-funded: https://uxarray.readthedocs.io/

- CMEMS Baltic bathymetry (for potential mesh-refinement sources): via Copernicus Marine Service.

- Existing OSMOSE-Python grid code: osmose/engine/grid.py, osmose/engine/processes/movement.py, ui/pages/grid_helpers.py, osmose/schema/grid.py.

Author’s note: this document is a scoping study, not a design spec. The next step — if pursued — is Phase 0 (the GridTopology abstraction), which has value on its own (cleaner code, easier testing) even if the non-rectangular work never happens.